# 4-BIT REGISTER MINI PROJECT

## Design (member a)

## 1. Input and Output Signals

| Signal | Description |
| --- | --- |
| Clk | Clock input |
| rst | Reset input |
| s1, s0 | Operation selection inputs |
| sin_right | Serial input during right shift |
| sin_left | Serial input during left shift |
| q[3:0] | Four-bit register output |

## 2. Function Table

| S1 | S0 | Operation | Description |
| --- | --- | --- | --- |
| 0 | 0 | Hold | Retains the current register value |
| 0 | 1 | Complement | Inverts all four register bits |
| 1 | 0 | Shift Right | Shifts bits toward the least significant bit |
| 1 | 1 | Shift Left | Shifts bits toward the most significant bit |

### Next-State Equations

Let Q3, Q2, Q1 and Q0 represent the current register bits. Let Q3⁺, Q2⁺, Q1⁺ and Q0⁺ represent their next values.

Hold — S1S0 = 00

- Q3⁺ = Q3

- Q2⁺ = Q2

- Q1⁺ = Q1

- Q0⁺ = Q0

Complement — S1S0 = 01

- Q3⁺ = NOT Q3

- Q2⁺ = NOT Q2

- Q1⁺ = NOT Q1

- Q0⁺ = NOT Q0

Shift Right — S1S0 = 10

- Q3⁺ = sin_right

- Q2⁺ = Q3

- Q1⁺ = Q2

- Q0⁺ = Q1

Shift Left — S1S0 = 11

- Q3⁺ = Q2

- Q2⁺ = Q1

- Q1⁺ = Q0

- Q0⁺ = sin_left

## 3. MUX Input Mapping

Each 4×1 multiplexer has four data inputs and two select inputs. The select inputs determine which data input reaches the output.

| S1 | S0 | Selected input |
| --- | --- | --- |
| 0 | 0 | I0 |
| 0 | 1 | I1 |
| 1 | 0 | I2 |
| 1 | 1 | I3 |

### Data Inputs for Each Multiplexer

| Multiplexer | I0 — Hold | I1 — Complement | I2 — Shift Right | I3 — Shift Left |
| --- | --- | --- | --- | --- |
| MUX Q3 | Q3 | Q3’ | sin_right | Q2 |
| MUX Q2 | Q2 | Q2’ | Q3 | Q1 |
| MUX Q1 | Q1 | Q1’ | Q2 | Q0 |
| MUX Q0 | Q0 | Q0’ | Q1 | sin_left |

## 4. Boolean Equations

The general Boolean equation for a 4×1 multiplexer is:

D = I0 S1’S0’+ I1 S1’S0 + I2 S1S0’+ I3S1S0

D3 = Q3 S1’ S0’+  Q3’ S1’S0 + sin_right·S1S0’+ Q2 S1S0

D2 = Q2 S1’ S0’ +  Q2’ S1’S0 + Q3 S1 S0’+ Q1 S1S0

D1 = Q1 S1’ S0’ +  Q1’ S1’S0 + Q2 S1S0’ + Q0 S1S0

D0 = Q0 S1’ S0’ +  Q0’ S1’S0 + Q1S1 S0’ + sin_left·S1S0

Here, D3, D2, D1 and D0 are the inputs to the four D flip-flops.

## 6. Block Diagram

## 7. circuit diagram